# CARAPACE

# Master Bill of Materials

Document ID: CPX-BOM-01
Revision: REV-A
Status: Engineering Draft

# 1. Procurement List

| Category | Component | Specification | Qty | Mandatory | Approx. Cost (₹) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| Controller | ESP32 DevKit V1 / STM32 Nucleo | Wi-Fi MCU | 1 | Yes | 500–900 | Developer may choose |
| Sensor | MPU6050 | 6-axis IMU | 1 | Yes | 200 | Motion sensing |
| Sensor | MAX30102 | PPG HR & SpO₂ | 1 | Yes | 250 | Optical sensor |
| Sensor | EDA Module | Analog GSR | 1 | Yes | 350 | Skin conductance |
| Communication | SIM800L | GSM | 1 | Yes | 450 | SMS |
| Communication | NEO-6M | GPS | 1 | Yes | 700 | Location |
| Deployment | CO₂ Cartridge | 16 g | 1 | Yes | 300 | Inflation source |
| Deployment | Solenoid Valve / Servo | Compatible | 1 | Yes | 400–800 | Trigger |
| Deployment | 5-Chamber Airbag | TPU-coated nylon | 1 | Yes | Custom | Prototype fabrication |
| Power | 18650 Cells | Li-ion | 2 | Yes | 600 | Matched pair |
| Power | 2S BMS | Protection board | 1 | Yes | 250 | Battery safety |
| Power | USB-C Charging Module | 2S compatible | 1 | Yes | 200 | Charging |
| Mechanical | PETG Enclosure | 3D printed | 1 | Yes | Custom | Housing |
| Mechanical | Silicone Tubing | 4–6 mm OD | 2 | Yes | 250 | Pneumatic routing |
| Mechanical | Push-fit Connectors | For tubing | 6 | Yes | 300 | Air connections |
| Assembly | JST Connectors | 2/3-pin | 10 | Yes | 250 | Modular wiring |
| Assembly | 22 AWG Silicone Wire | Power | 2 | Yes | 200 | Power wiring |
| Assembly | 28 AWG Silicone Wire | Signal | 5 | Yes | 200 | Signal wiring |
| Assembly | M2/M3 Screws & Inserts | Assorted | 1 | Yes | 300 | Fastening |

# 2. Estimated Prototype Cost

Target budget: ₹10,000 to ₹12,000
Estimated electronics, mechanical parts, and consumables fall within the approved prototype budget, excluding fabrication/service charges from the external hardware developer.

# 3. Procurement Notes

• Equivalent components may be substituted only if functionality is maintained.

• All substitutions must be approved before integration.

• All electronic modules shall be tested individually before assembly.

• CO₂ cartridge compatibility shall be verified with the selected trigger mechanism.