# CARAPACE

# Electrical Connections & Wiring Specification

Document ID: CPX-ELE-01
Revision: REV-A
Status: Engineering Draft

# 1. System Power Distribution

2×18650 Battery Pack → 2S BMS → Main Power Switch → 5V/3.3V Regulators → MCU and Modules.
The CO₂ trigger actuator shall receive power through a dedicated controlled output from the MCU.

# 2. Module Wiring

| Source | Destination | Interface | Notes |
| --- | --- | --- | --- |
| Battery Pack | 2S BMS | Power | Primary supply |
| 2S BMS | Main Switch | Power | Protected output |
| Main Switch | ESP32/STM32 | 5V/3.3V | Controller supply |
| ESP32/STM32 | MPU6050 | I²C | Shared bus |
| ESP32/STM32 | MAX30102 | I²C | Shared bus |
| EDA Sensor | ESP32/STM32 | ADC | Analog input |
| ESP32/STM32 | SIM800L | UART | SMS |
| ESP32/STM32 | NEO-6M | UART | GPS |
| ESP32/STM32 | Solenoid/Servo | GPIO/PWM | CO₂ trigger |
| ESP32/STM32 | Status LED | GPIO | System indication |
| ESP32/STM32 | Buzzer | GPIO | Audible alert |

# 3. Wiring Requirements

• Use 22 AWG silicone wire for power distribution.

• Use 26–28 AWG silicone wire for signal wiring.

• Use JST connectors for all removable modules.

• Route pneumatic tubing separately from electrical wiring.

• Secure wiring with cable ties or adhesive clips to prevent movement.

# 4. Electrical Verification Checklist

☐ Verify battery polarity before power-up.

☐ Verify regulator output voltage.

☐ Check I²C communication with all sensors.

☐ Check UART communication with GSM and GPS modules.

☐ Verify GPIO output to deployment actuator.

☐ Perform continuity test before installing the CO₂ cartridge.