# CARAPACE

# Hardware Build Specification

Document ID: CPX-HBS-01
Revision: REV-C
Status: Engineering Draft

# 10. Recommended Pin Mapping (ESP32 Option)

| Device | Interface | Suggested Connection |
| --- | --- | --- |
| MPU6050 | I²C | SDA GPIO21, SCL GPIO22 |
| MAX30102 | I²C | Shared I²C Bus |
| EDA Sensor | Analog | GPIO34 (ADC) |
| SIM800L | UART | TX2/RX2 |
| NEO-6M GPS | UART | TX1/RX1 (or Software UART) |
| Solenoid / Servo | GPIO Output | PWM-capable GPIO |
| Status LED | GPIO Output | Any digital GPIO |
| Buzzer | GPIO Output | Any digital GPIO |

# 11. Final Bill of Materials

| Item | Qty | Mandatory | Approx. Cost (₹) | Remarks |
| --- | --- | --- | --- | --- |
| ESP32 / STM32 | 1 | Yes | 500-900 | Controller |
| MPU6050 | 1 | Yes | 200 | IMU |
| MAX30102 | 1 | Yes | 250 | PPG |
| EDA Sensor | 1 | Yes | 350 | GSR |
| SIM800L | 1 | Yes | 450 | GSM |
| NEO-6M | 1 | Yes | 700 | GPS |
| 16 g CO₂ Cartridge | 1 | Yes | 300 | Inflation |
| Solenoid Valve / Servo | 1 | Yes | 400-800 | Trigger |
| TPU Airbags | 5 | Yes | Custom | Protection |
| 2×18650 Cells | 2 | Yes | 600 | Battery |
| 2S BMS | 1 | Yes | 250 | Battery protection |
| USB-C Charging Module | 1 | Yes | 200 | Charging |
| PETG Parts | 1 | Yes | Custom | 3D printed |
| JST Connectors, Wires, Fasteners | Set | Yes | 500 | Assembly |

# 12. Prototype Acceptance Checklist

☐ All sensors produce valid readings.

☐ Battery charges and powers the system.

☐ GSM module sends SMS successfully.

☐ GPS module provides location fix.

☐ Detection logic executes without crashes.

☐ Deployment actuator operates correctly.

☐ CO₂ trigger functions safely (bench-tested).

☐ Prototype runs continuously for at least 4 hours.

☐ Complete source code and wiring diagram delivered.